Transport System Summary

计算部分

Part 1 Public Transport

Service headway: The time between vehicles past a given point.

Service frequency: The number of vehicles per unit time past a certain point on a route.

Service capacity: The maximum number of passengers that can be carried past a certain point on a fixed route.

- Expected waiting time is h/2

Frequency determination

Policy headway (max headway): minimum level of service frequency.

Minimum headway: maximum frequency (for bus and rail system, based on the capacity limitations)

Load management: managing load at the peak load point along the route.

Optimal frequency(cost-minimizing): minimum total system cost.

total system cost = Cost (operation) + Cost (total waiting time)

= C* T* f + VOT* p* (expected waiting time: h/2)

= C* T* f + (VOT* p)/2f

Timetable development

Required fleet size

Nv = T ∙ f

T: route cycle time (running time both ways + layovers at terminals)

f: frequency

Part 2 Traffic Flow

Flow (Volume)

Headway

Density

Spacing

q = k * u

u: avg traffic speed (km/h)

Green-shield Model

Level of service

| Level | A | B | C | D | E | F |
| --- | --- | --- | --- | --- | --- | --- |
| Density | 0-7 | 7.1-11 | 11.1-16 | 16.1-22 | 22.1-28 | >28 |

Part 3 Trip Distribution

O-D Matrix

Gravity Model

→ Trip Deterrence Function

The impact of travel impendence in destination choice

Calibrate the correction factors

Part 4 Model Choice

Utility model

Logit Model

Maximum Likelihood Estimation (MLE)

Likelihood of an observed choice is the predicted probability of the chosen mode.

Likelihood of a sample is the product of likelihood of each observed choice in the sample.

Part 5 Network Assignment

Volume-Delay Function (VDF)

Incremental Assignment algorithm

Method of successive averages (MSA)